Amennyiben probléma lenne a magas bitrátájú állomások lejátszásával:

Forrás: https://4pda.to/forum/index.php?showtopic=1010378&st=16700#entry132067099

Régóta próbálom kideríteni, hogy miért működnek egyes internetes állomások megszakításokkal és állandó rángatásokkal.

A cél a zajmentes hangzás elérése volt. Például: "Iskatel 320 aac" http://iskatel.hosting…u:8015/iskatel-320.aac

E-rádión ESP32-Wrover + VS1053B / I2S-sel.

Hetekig próbálkoztam ezzel, és megtaláltam a megoldást!

Wroveren, sőt Wroomon is működött interferencia és rángatás nélkül.

Nem a rossz internetről van szó, hanem az Arduino könyvtárakról .

Egy kis háttértörténet:

A yoRadio Arduinón van lefordítva, és az Arduino már lefordított ESP-IDF-et ( https://github.com/espressif/esp-idf ) használ sok könyvtárral .

Az ESP-IDF fordításakor bármit konfigurálhatsz, de az Arduino az alapértelmezett konfigurációt használja, és nincs mód a javításra.

Találtam egy jó megoldást a https://github.com/sch…Better_WiFi_throughput oldalon . Az internettel kapcsolatos probléma az ESP-IDF SDK LWIP könyvtára által használt maximális TCP kapcsolatok számában volt.

Újrafordítottam az ESP-IDF-et új LWIP paraméterekkel:

CONFIG_LWIP_MAX_ACTIVE_TCP=512

CONFIG_LWIP_MAX_LISTENING_TCP=512

CONFIG_LWIP_TCP_SND_BUF_DEFAULT=8192

CONFIG_LWIP_TCP_WND_DEFAULT=32768

CONFIG_LWIP_TCP_RECVMBOX_SIZE=32

CONFIG_LWIP_TCP_QUEUE_OOSEQ=y,

hogy megkapjam az új liblwip.a könyvtárat, és lecseréljem azt az Arduino-ban a régi helyett a 2.0.X kernelhez.

A liblwip cseréjének lépései:

1. Fájlok helye: Audio_IDF_MOD\CORE_3.3.3-IDF_ver5.5

2. Másold át a liblwip.a és libesp_netif.a fájlt az alábbi könyvtárba:

ESP32 modulnál: használd az Audio_IDF_MOD\CORE_3.3.3-IDF_ver5.5\ESP32 könyvtárban lévő fájlokat.

Másolás célja:

c:\Users\<username>\AppData\Local\Arduino15\packages\esp32\tools\esp32-arduino-libs\idf-release_v5.5-f1a1df9b-v3\esp32\lib\

ESP32-S3 modulnál használd az Audio_IDF_MOD\CORE_3.3.3-IDF_ver5.5\ESP32-S3\liblwip_idf.5.5_3.3.3_kernel\ mappában lévő 3 könyvtár bármelyikét. (Én a light verziót használom, nekem az vált be a legjobban (számomra a legstabilabb), de mindhárom verzió jó lehet, más-más buffer méretekkel.
Másolás célja:

c:\Users\<username>\AppData\Local\Arduino15\packages\esp32\tools\esp32-arduino-libs\idf-release_v5.5-f1a1df9b-v3\esp32s3\lib\

értelemszerűen a username a saját felhasználói könyvtárad

(ebben a könyvtárban lennie kellett a régi liblwip.a és libesp_netif.a fájlnak, amit lecserélünk az új fájllal)

3. Indítsd újra az Arduinót

4. Fordítsd újra az E-radiót, mint korábban, és töltsd be az ESP32-be

If you have problems playing high-bitrate stations:

Source: https://4pda.to/forum/index.php?showtopic=1010378&st=16700#entry132067099

I've been trying for a long time to figure out why some internet radio stations play with interruptions and constant stuttering.
My goal was to achieve clean, noise-free audio playback. For example:
"Iskatel 320 aac" http://iskatel.hosting…u:8015/iskatel-320.aac
on E-radio using ESP32-Wrover + VS1053B / I2S.

I experimented with this for weeks and finally found the solution!
It worked smoothly, without interference or stuttering, both on Wrover and even on Wroom modules.
The problem is not with a poor internet connection, but with the Arduino libraries.

Here’s a bit of background:

yoRadio is compiled on Arduino, and Arduino itself uses a precompiled version of ESP-IDF (https://github.com/espressif/esp-idf) along with many libraries.
When you compile ESP-IDF yourself, you can configure everything, but Arduino always uses the default configuration and there is no way to change or fix it directly.

I found a good solution on https://github.com/sch…Better_WiFi_throughput.
The issue with the internet connection was caused by the maximum number of TCP connections set by the LWIP library in the ESP-IDF SDK.

So, I recompiled ESP-IDF with new LWIP parameters:

CONFIG_LWIP_MAX_ACTIVE_TCP=512

CONFIG_LWIP_MAX_LISTENING_TCP=512

CONFIG_LWIP_TCP_SND_BUF_DEFAULT=8192

CONFIG_LWIP_TCP_WND_DEFAULT=32768

CONFIG_LWIP_TCP_RECVMBOX_SIZE=32

CONFIG_LWIP_TCP_QUEUE_OOSEQ=y

to get a new liblwip.a library, which I then replaced in Arduino, overwriting the old one for the 2.0.X core.

How to replace the liblwip library:

- File location: Audio_IDF_MOD\CORE_3.3.3-IDF_ver5.5

- Copy the liblwip.a and libesp_netif.a files to the following folder:

For ESP32 modules: Use the files in the Audio_IDF_MOD\CORE_3.3.3-IDF_ver5.5\ESP32 directory.

Target folder:
c:\Users\<username>\AppData\Local\Arduino15\packages\esp32\tools\esp32-arduino-libs\idf-release_v5.5-f1a1df9b-v3\esp32\lib\

For ESP32-S3 modules: Use the files in the Audio_IDF_MOD\CORE_3.3.3-IDF_ver5.5\ESP32-S3\liblwip_idf.5.5_3.3.3_kernel\ directory. (I use the light version, which works best for me (it's the most stable), but all three versions can be good, with different buffer sizes.)

Target folder:
c:\Users\<username>\AppData\Local\Arduino15\packages\esp32\tools\esp32-arduino-libs\idf-release_v5.5-f1a1df9b-v3\esp32s3\lib\

Obviously, replace "user" with your own Windows username.
(In this directory, there should already be an old liblwip.a file—overwrite it with the new one.)

- Restart Arduino.

- Recompile your E-radio project as usual, and upload it to your ESP32.